Understanding GPUs

https://medium.com/ai-insights-cobet/understanding-gpu-architecture-basics-and-key-concepts-40412432812b

What is a GPU

- GPUs can best be described as an inverse to CPUs

    - Instead of running complex tasks sequentially, GPUs run smaller tasks across thousands of cores simultaneously

  - CPUs can have limited parallelism, but not nearly to the extent of GPUs

Memory Hierarchy

- Split across 3 different sources:

  - SRAM

    - Very fast but small cache memory

    - Located inside GPU, uses Registers, L1 Cache, and L2 Cache

      - L1 Cache

        - First level cache inside Streaming Multiprocessor (SM)

        - Stores frequently accessed data to speed up calculations

      - L2 Chache

        - Larger, second level cache shared between multiple SMs

        - Helps store and reuse data too big for L1 cache

    - Caches reduce reliance on external memory (VRAM) and speed up calculations

    - Optimizing how data is stored speeds up GPU processing

    - Most important for getting the fundamentals of GPU processing working

  - HBM

    - Used in High Performance GPUs for AI and deep learning workloads

    - Stacks memory vertically

    - Overall, lowest priority for a GPU of the scale envisioned

  - DRAM

    - Main memory

    - Found on video card

    - Stores large amounts of data

    - Good to work on once main GPU is figured out

- Memory Transfer

  - Computationally expensive to transfer data from DRAM to SRAM

  - Optimizing efficient memory usage optimizes performance of tasks

Streaming Processors (SMs)

- Fundamental Processing Unit of a GPU

- Each SM contains multiple GPU cores, a small memory pool (SRAM), and execution units

- Each SM operates independently

- GPU computational power directly tied to number of SMs

- Project should implement multiple SMs in parallel

- Programs:

  - Specific instances of kernel code running in parallel

  - Split between SMs

    - Number split between each SM depends on available memory in SM, and memory required by Program

- GPU Cores:

  - Smallest unit of compute

  - Optimized for floating point operations

    - Performs 1 FLOP per cycle

- Warps:

  - Groups of GPU cores

  - All cores in a warp must execute the same instruction simultaneously, but operate on different data